Operator Troubleshooting Checklist: Rainbow

| Issue | Symptoms | Troubleshooting steps | When to call maintenance |
| --- | --- | --- | --- |
| Visible damage / misalignment | - Bent or broken holder arms, brackets, or other structural components. - Safety light barriers or fences are bent, knocked out of alignment, or damaged - Controller column is bent - Loose or missing parts like blocks, bolts, or carriage guides - Stacks of sheets sitting unevenly or "on another stack" due to misalignment | - Visually inspect Carefully walk around the machine and look for any visibly bent, broken, loose, or missing parts, especially after a known impact or crash - Check safety light barriers If a light barrier is simply out of alignment (not broken), try to gently push it back into its correct position - Check fence sections If a safety fence section is open or slightly bent, try to gently push it back into place, ensuring any links or latches engage | - If you find any bent or broken structural components (e.g., holder arms, large brackets, main beams) - If safety light barriers or fences are significantly damaged, cracked, or cannot be easily realigned - If you find loose hardware that you cannot safely access or tighten - If the machine continues to operate with visible damage, this can lead to further, more severe problems |
| Machine Not Referencing, Not Turning On, or X-Axis Issues | - Machine fails to reference or gets stuck during referencing - Machine makes jerking movements or strange sounds when trying to reference - Error messages related to X-axis, drive unit trip, or "cross rail raised" - Machine won't turn on at all - Machine stops mid-reference | - Check for obstructions Carefully look along the X-axis rail for any debris, tools, or anything that might be physically blocking the crane's movement. - Reboot - Turn the main power knob to the OFF position - Wait at least 5 minutes - Turn the main power knob back ON - Try referencing the machine again - Check Breakers If the machine won't turn on at all, or if you suspect a power issue, check the main breaker for the Rainbow machine. If it's tripped, reset it. (Only if safe and accessible, and you know which breaker it is, otherwise, call maintenance) - Reflectors Locate the laser reflectors on the machine's rails. Gently wipe them clean with a soft, dry cloth to remove any dust or black smears | - If the machine still won't reference or turn on after these steps - If you hear loud grinding, snapping, or abnormal noises - If the machine jerks violently or doesn't move smoothly - If you see any visible damage to the rails, drives, or wiring - If the X-axis error persists or you get specific fault codes you don't understand |
| Incorrect Sheet Handling / Suction Problems | - TLF not picking up sheets correctly - Suction cups are only partially on the sheet or in the air - Sheets fall off the machine or slip, causing stacks to be messy - Machine gets "stuck on vacuum" with or without a sheet - Machine will not drop off a sheet or crashes into back stops | - Recalibrate the machine If sheets are not being picked up or dropped off correctly, try running the machine's calibration sequence as per your standard operating procedure - Check for obstructions/debris Ensure the suction cups are clean and free of any debris or plastic sheeting that might prevent a proper seal - Clear stuck sheets/vacuum If the machine is "stuck on vacuum" or has dropped a sheet, manually remove any sheets causing the issue and reset the machine. | - If calibration does not resolve the issue - If the machine consistently fails to pick up or drop off sheets correctly - If the machine repeatedly gets stuck on vacuum - If you suspect a sensor is faulty or there's internal damage from a crash |
| Belt Monitoring Error / Suction Pipe / Bracket Issues | - Error message "Belt monitoring error" displayed - Machine stops unexpectedly - Belt appears loose - Suction is not coming on when picking up a sheet | - Check for loose cables Inspect the cables in the area where suction cups are located, ensuring they are securely connected - Visual inspection of suction brackets Look for any visibly bent or displaced brackets/beams that hold the suction pipes or cups, especially in the middle section. | - If the "Belt monitoring error" persists after checking for loose cables - If you find any bent, cracked, or severely misaligned brackets or beams supporting the suction system - If there is no suction when picking up sheets, even after checking for obvious debris. |
| Measurement Errors / Width Stop Errors | - Machine gives errors related to length or width measurement - "Width stop error" message - Sheets not being picked up correctly due to incorrect width measurement (suction cups off) | - Clean measurement areas Gently clean any visible sensors or rods associated with length and width measurement with a soft, dry cloth - Recalibrate (if applicable) If the issue affects sheet pickup, try recalibrating the machine (as per 'Incorrect Sheet Handling') | - If cleaning or recalibration does not resolve the measurement errors - If errors persist or "width stop error" messages continue - If you notice any air leaks (hissing sounds) around pneumatic cylinders |
| Miscellaneous Errors | - "Empty slot X" or "slot not in position" error message - Machine displaying "Axis not in position width" error - Audible air leaks (hissing sounds) from the machine | - Check for air leaks Listen for any hissing sounds indicating an air leak - Verify slot status If an "empty slot" error appears, ensure the designated slot is indeed empty or cleared - Check for detached cables Visually inspect areas around sensors for any cables that might have come loose or detached, especially if an "Axis not in position" error occurs | - If you detect any air leaks - If "empty slot" errors persist even after verifying the slot is clear (e.g., if a sensor is stuck) - If you find a detached sensor cable and are unsure how to safely reattach it, or if reattaching it does not resolve the issue |

Resources for maintenance

| Issue | Related Work Order #’s | Parts often needed (if any) |
| --- | --- | --- |
| Visible damage / misalignment | 2321, 3424, 7931, 7950, 9253, 10219, 10891, 12493, 12657, 13929, 15319, 18930 | Arm holder (30-246-76581), Block (25-520-236691), Button holder (25-310-29260), Bolt (38-313-44891), Bracket holder (30-246-76581 / MFG 2-406-76-5811), Fixing device (25-206-06060), Guide carriage (25-406-04598) |
| Machine Not Referencing, Not Turning On, or X-Axis Issues | 8045, 8068, 9349, 9668, 10159, 14421, 14794, 19844, 14153, 2412 (2412 involved extensive repairs, requiring specialized technician involvement beyond operator scope.) | No operator-replaceable parts; issues often require technician diagnosis and specialized parts like drives, sensors, cables |
| Incorrect Sheet Handling / Suction Problems | 2109, 5922, 7676, 13239, 13818, 14959 | Sensor (IE5338), (Specific encoder/width sensor part number not provided in these WOs). |
| Belt Monitoring Error / Suction Pipe / Bracket Issues | 6874, 7321, 12513, 12798, 14963 | Middle fixing device (25-206-66120), Center beams (25-406-66110). |
| Measurement Errors / Width Stop Errors | 4527, 2109, 12876 | Cylinder (25-035-12892), Sensor/Encoder (generic). |
| Miscellaneous Errors | 3037, 6489, 16138 | Air regulator (generic), (specific sensor/cable parts not provided in these WOs). |